- CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

| Product Name | Eastman(TM) Cellulose Acetate Butyrate (CAB-381-0.5) |
| --- | --- |
| Product Identification Number(s) | 01873-00, P01873AB, P0187300, P0187301, P0187302, P0187303, P0187332, P0187333, E0187301 |
| Manufacturer/Supplier | Eastman Chemical Company 200 South Wilcox Drive Kingsport, TN 37660-5280 US +14232292000 |
| MSDS Prepared by | Eastman Product Safety and Health |
| Chemical Name | not applicable |
| Synonym(s) | 321875 |
| Molecular Formula | not applicable |
| Molecular Weight | not applicable |
| Product Use | polymer |
| OSHA Status | nonhazardous |

For emergency health, safety, and environmental information:  telephone (86) 532 83889090 in China;  60-9-583-9696 in Malaysia;  (65) 6831-3233 in Singapore and other Asia Pacific regions; or 00-1-423-229-4511 in the United States.

For emergency transportation information:  telephone (86) 532 83889090 in China;  (65) 6831-3233 in Singapore and other Asia Pacific regions; or 00-1-423-229-4511 in the United States.  Identify the call as a transportation emergency.

- COMPOSITION INFORMATION ON INGREDIENTS

(Typical composition is given, and it may vary.  A certificate of analysis can be provided, if available.)

| Weight % |  | Component | CAS Registry No. |  |
| --- | --- | --- | --- | --- |
| 100% | cellulose acetate butyrate |  |  | 9004-36-8 |

- HAZARDS IDENTIFICATION

CAUTION!

POWDERED MATERIAL MAY FORM EXPLOSIVE DUST-AIR MIXTURES

| HMIS® Hazard Ratings: | Health - 1, Flammability -1, Chemical Reactivity - 0 |
| --- | --- |

HMIS® rating involves data interpretations that may vary from company to company. They are intended only for rapid, general identification of the magnitude of the specific hazard. To deal adequately with the safe handling of this material, all the information contained in this MSDS must be considered.

- FIRST-AID MEASURES

Inhalation: If symptomatic, move to fresh air. Get medical attention if symptoms persist.

Eyes: Any material that contacts the eye should be washed out immediately with water. If easy to do, remove contact lenses. Get medical attention if symptoms persist.

Skin: Wash with soap and water. Get medical attention if symptoms occur.

Ingestion: Seek medical advice. Material is not expected to be absorbed from the gastrointestinal tract so that induction of vomiting should not be necessary.

- FIRE FIGHTING MEASURES

Extinguishing Media: water spray, dry chemical, carbon dioxide

Special Fire-Fighting Procedures: Wear self-contained breathing apparatus and protective clothing.

Hazardous Combustion Products: carbon dioxide, carbon monoxide

Unusual Fire and Explosion Hazards: Powdered material may form explosive dust-air mixtures.

Sensitivity to Static Discharge: Material may accumulate a static charge which could act as an ignition source.

- ACCIDENTAL RELEASE MEASURES

Sweep or scoop up and remove.

- HANDLING AND STORAGE

Personal Precautionary Measures: No special precautionary health measures should be needed under anticipated conditions of use.

Prevention of Fire and Explosion: Keep from contact with oxidizing materials.  Mixing cellulose esters in a nonpolar hydrocarbon, such as toluene or xylene, may result in the buildup of static electricity, which can cause  a flash fire or an explosion. When adding cellulose ester to any flammable liquid, an inert gas atmosphere should be maintained within the vessel.  Minimize dust generation and accumulation.  In the United States of America, refer to NFPA® Pamphlet No. 654, "Prevention of Fire and Dust Explosions in the Chemical, Dye, Pharmaceutical, and Plastics Industries."

Storage: Keep container closed.

- EXPOSURE CONTROLS/PERSONAL PROTECTION

Country specific exposure limits have not been established or are not applicable unless listed below.

Ventilation: Good general ventilation (typically 10 air changes per hour) should be used. Ventilation rates should be matched to conditions. Supplementary local exhaust ventilation, closed systems, or respiratory and eye protection may be needed in special circumstances; such as poorly ventilated spaces, heating, evaporation of liquids from large surfaces, spraying of mists, mechanical generation of dusts, drying of solids, etc.

Respiratory Protection: If engineering controls do not maintain airborne concentrations below recommended exposure limits (where applicable) or to an acceptable level (in countries where exposure limits have not been established), an approved respirator must be worn. In the United States of America, if respirators are used, a program should be instituted to assure compliance with OSHA Standard 63 FR 1152, January 8, 1998. Respirator type: Air-purifying respirator with an appropriate, government approved (where applicable), air-purifying filter, cartridge or canister. Contact health and safety professional or manufacturer for specific information.

Eye Protection: It is a good industrial hygiene practice to minimize eye contact.

Skin Protection: It is a good industrial hygiene practice to minimize skin contact.

Recommended Decontamination Facilities: eye bath, washing facilities

- PHYSICAL AND CHEMICAL PROPERTIES

| Physical Form: solid (powder) |  |  |
| --- | --- | --- |
| Color: white |  |  |
| Odor: slight, characteristic |  |  |
| Specific Gravity: > 1estimated |  |  |
| Melting Point: 127 - 240 °C |  |  |
| Solubility in Water: negligible |  |  |
| Flash Point: not applicable, combustible solid |  |  |
| Thermal Decomposition Temperature: Thermal stability not tested. Low stability hazard expected at normal operating temperatures. |  |  |
| Dust Explosion Class: St 2 - strong explosion |  |  |

- STABILITY AND REACTIVITY

| Stability: | Not fully evaluated. Materials containing similar structural groups are normally stable. |  |
| --- | --- | --- |
| Incompatibility: | Material reacts with strong oxidizing agents. |  |
| Hazardous Polymerization: | Will not occur. |  |

- TOXICOLOGICAL INFORMATION

Acute toxicity data, if available, are listed below. Additional toxicity data may be available on request.

| Oral LD-50:(rat) | >6,400 mg/kg(highest dose tested) |  |
| --- | --- | --- |
| Dermal LD-50: ( guinea pig) | > 1,000 mg/kg (highest dose tested) |  |
| Skin Irritation (guinea pig) | slight |  |
| Skin Sensitization: (guinea pig) |  | none |

- ECOLOGICAL INFORMATION

Acute toxicity data, if available, are listed below. Additional toxicity data may be available on request.

This material has not been tested for environmental effects.

- DISPOSAL CONSIDERATIONS

Discharge, treatment, or disposal may be subject to national, state, or local laws. Incinerate.

- TRANSPORT INFORMATION

Important Note:  Shipping descriptions may vary based on mode of transport, quantities, package size, and/or origin and destination. Consult your company's Hazardous Materials/Dangerous Goods expert for information specific to your situation.

Sea - IMDG (International Maritime Dangerous Goods)

Class  not regulated

Air - ICAO (International Civil Aviation Organization)

Class  not regulated

- REGULATORY INFORMATION

SARA 313: none, unless listed below

Carcinogenicity Classification (components present at 0.1% or more):  none, unless listed below

TSCA (US Toxic Substances Control Act): This product is listed on the TSCA inventory. Any impurities present in this product are exempt from listing.

DSL (Canadian Domestic Substances List) and CEPA (Canadian Environmental Protection Act): This product is listed on the DSL. Any impurities present in this product are exempt from listing.

EINECS (European Inventory of Existing Commercial Chemical Substances):   All components of this product are listed on EINECS. Any polymer present in this product has regulatory clearance under Directives of the European Union.

AICS / NICNAS (Australian Inventory of Chemical Substances and National Industrial Chemicals Notification and Assessment Scheme): This product is listed on AICS or otherwise complies with NICNAS.

MITI (Japanese Handbook of Existing and New Chemical Substances):  This product is listed in the Handbook or has been approved in Japan by new substance notification.

ECL (Korean Toxic Substances Control Act):   This product is listed on the Korean inventory or otherwise complies with the Korean Toxic Substances Control Act.

- OTHER INFORMATION

Visit our website at www.EASTMAN.com or call 001-423-229-2000.

The information contained herein is based on current knowledge and experience; no responsibility is accepted that the information is sufficient or correct in all cases. Users should consider these data only as a supplement to other information. Users should make independent determinations of suitability and completeness of information from all sources to assure proper use and disposal of these materials, the safety and health of employees and customers, and the protection of the environment.

Highlighted areas indicate new or changed information.